WAVLON LASERS

TubeCut — Fiber Laser Tube Cutting Full Series Reference

TubeCut Double Chuck & TubeCut Triple Chuck (Heavy Duty) Model Lineup

Engineering reference benchmarked against LD Laser (LONGDIAO Intelligent Equipment) T5 / T8 Series

Internal Document — Wavlon Lasers Inc.

Toronto, Ontario, Canada

# 1. Executive Summary

This document defines Wavlon Lasers' full fiber laser tube/pipe cutting line-up — branded TubeCut, consistent with the PowerCut / ProCut / UltraCut sheet-cutting naming family — organized into two chuck-count series and four market tiers: Entry, Intermediate, and Heavy Duty. The line-up is engineered with reference to LD Laser's 2026 T5 (double-chuck) and T8 (triple-chuck) catalog series, which were used as the technical and structural benchmark.

### 1.1 Series Overview

| Series | Chuck Count | Tier(s) | Power Range | Diameter Range | Bed Length |
| --- | --- | --- | --- | --- | --- |
| TubeCut Double Chuck | 2 (Double) | Entry + Intermediate | 1.5 – 12 kW | 10 – 360 mm | 6,000 mm |
| TubeCut Triple Chuck | 3 (Triple) | Heavy Duty | 6 – 30 kW | 10 – 680 mm (round) | 12,000 mm |

Note: square-tube capacity is generally equal to round-tube capacity per model, except the top TTC-680 tier, where the square-chuck maximum is 570mm (vs. 680mm round) due to chuck-jaw stroke geometry.

# 2. Model Naming Convention

Wavlon TubeCut models follow the pattern:

T[DC/TC]-[Max Diameter mm]-[Power W]

- TubeCut — Wavlon's fiber laser tube/pipe cutting brand, alongside PowerCut, ProCut, and UltraCut on the sheet-cutting side

- DC — Double Chuck series (Entry & Intermediate tiers)

- TC — Triple Chuck series (Heavy Duty tier)

- [Max Diameter mm] — maximum round-tube cutting diameter in millimetres

- [Power W] — laser source power in watts, e.g. 4000 = 4kW (omitted when referring to a diameter tier generically, e.g. “TDC-160”)

Example: TDC-160-4000 = TubeCut Double Chuck, 160mm max diameter, 4,000W configuration.

# 3. TubeCut Double Chuck Series (Entry & Intermediate)

Reference benchmark: LD Laser T5 Series, 1.5kW–12kW, double chuck, ±0.05mm repeat positioning accuracy, ≈1.2G acceleration, optional processing diameters 6012/6016/6024/6036 (120/160/240/360mm).

### 3.1 Model Lineup

| Model | Tier | Max ⌀ Round | Max ⌀ Square | Power Options | Bed Length | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| TDC-120 | Entry | 120 mm | 120 mm | 1.5 / 2 / 3 kW | 6,000 mm |  |
| TDC-160 | Entry | 160 mm | 160 mm | 2 / 3 / 4 / 6 kW | 6,000 mm | ⭐ Popular |
| TDC-240 | Intermediate | 240 mm | 240 mm | 4 / 6 / 8 kW | 6,000 mm | ⭐ Popular |
| TDC-360 | Intermediate | 360 mm | 360 mm | 6 / 8 / 12 kW | 6,000 mm |  |

### 3.2 Technical Parameters

| Parameter | TDC-120 | TDC-160 | TDC-240 | TDC-360 |
| --- | --- | --- | --- | --- |
| Max diameter (round / square) | 10–120mm | 10–160mm | 10–240mm | 10–360mm |
| Repeat positioning accuracy | ±0.05mm | ±0.05mm | ±0.05mm | ±0.05mm |
| Acceleration | ≈1.2G | ≈1.2G | ≈1.2G | ≈1.2G |
| Min. tail length | ≈40mm | ≈40mm | ≈40mm | ≈40mm |
| Pipe loading length | ≤6,050mm | ≤6,050mm | ≤6,050mm | ≤6,050mm |
| Max loading weight | 100 kg* | 150 kg* | 250 kg* | 400 kg* |
| Machine dimension (L×W×H) | 8,890×1,720×2,175mm* | 9,200×1,900×2,300mm† | 9,800×2,200×2,500mm† | 10,400×2,500×2,800mm† |
| Gross weight (approx.) | ≈2,500 kg* | ≈3,200 kg† | ≈4,400 kg† | ≈6,000 kg† |

* Grounded in Wavlon/LD reference spec sheet (S5-6012-3000W datasheet). † Indicative figures, scaled from the grounded 120mm/3kW datasheet — confirm exact dimensions/weights with engineering prior to quotation.

### 3.3 Entry Tier — TDC-120 / TDC-160

Designed for small-to-mid job shops entering tube laser cutting. Lower power options (1.5–6kW) keep capital and running costs down while covering the most common round/square/rectangular tube sizes used in furniture, railing, HVAC ducting, and light structural fabrication.

### 3.4 Intermediate Tier — TDC-240 / TDC-360

Targets growing fabrication shops needing larger diameter capacity and higher throughput. TDC-240 and TDC-360 are the most frequently quoted configurations in this series, balancing diameter capacity against machine footprint and price.

# 4. TubeCut Triple Chuck Series (Heavy Duty)

Reference benchmark: LD Laser T8 Series, 6kW–30kW, triple chuck, ±0.05mm repeat positioning accuracy, optional processing diameters 12036/12052/12064/12083 (360/520/640/830mm) at 6m or 12m bed length.

### 4.1 Model Lineup

| Model | Tier | Max ⌀ Round | Max ⌀ Square | Power Options | Bed Length | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| TTC-360 | Heavy Duty | 360 mm | 360 mm | 6 / 8 / 12 kW | 12,000 mm | ⭐ Popular |
| TTC-520 | Heavy Duty | 520 mm | 520 mm | 12 / 16 / 20 kW | 12,000 mm | ⭐ Popular |
| TTC-680 | Heavy Duty (Max) | 680 mm | 570 mm | 20 / 24 / 30 kW | 12,000 mm | Top tier |

### 4.2 Technical Parameters

| Parameter | TTC-360 | TTC-520 | TTC-680 / 570 |
| --- | --- | --- | --- |
| Max diameter (round / square) | 10–360mm | 10–520mm | 10–680 / 10–570mm |
| Repeat positioning accuracy | ±0.05mm | ±0.05mm | ±0.05mm |
| Acceleration | ≈1.0G† | ≈1.0G† | ≈1.0G† |
| Bed / pipe loading length | 12,000mm | 12,000mm | 12,000mm |
| Max loading weight | 800 kg† | 1,500 kg† | 2,500 kg† |
| Machine dimension (L×W×H) | ≈14,800×2,400×2,600mm† | ≈14,800×2,800×3,000mm† | ≈14,800×3,200×3,400mm† |
| Gross weight (approx.) | ≈7,500 kg† | ≈10,500 kg† | ≈14,000 kg† |

† All TubeCut Triple Chuck dimensional/weight/acceleration figures are indicative engineering estimates pending confirmation — no Wavlon/LD source datasheet exists yet for the triple-chuck heavy duty tier at the time of writing.

### 4.3 Heavy Duty Tier — TTC-360 / TTC-520

The most popular heavy-duty configurations, suited to structural steel fabricators, agricultural equipment manufacturers, and steel service centers processing large round and square HSS sections at 12m bed length.

### 4.4 Maximum Capacity Tier — TTC-680 (Round) / TTC-570 (Square)

The flagship configuration of the line-up. Maximum round-tube capacity is 680mm; maximum square-tube capacity is 570mm on the same triple-chuck platform, reflecting the difference between round bore clamping and square-jaw stroke geometry at full chuck extension.

Open item: LD Laser's own T8 catalog lists round-only diameter codes of 360 / 520 / 640 / 830mm. Wavlon's flagship tier (680mm round / 570mm square) was specified directly by Wavlon management and does not map 1:1 onto an LD code — closest LD reference points are bracketed between codes 12064 and 12083. Confirm final flagship diameter target before locking supplier sourcing.

# 5. Standard Componentry & Brand Sourcing

Applies across the full TubeCut Double Chuck and TubeCut Triple Chuck line-up unless a specific model calls for a substitution (e.g., higher-capacity chillers or reducers at the 30kW tier).

| Component | Brand / Model | Origin | Quantity |
| --- | --- | --- | --- |
| Laser Source | Raycus / MAX | China | 1 set |
| Laser Head (Tube) | BOCI BLT3T / BLT4T Series | China | 1 set |
| Control System | BOCHU FSCUT3000 | China | 1 set |
| Rotary Pneumatic Chuck | BEUT | China | 1 set per chuck |
| X/Y/Z Axis Linear Rail | SHAC | China | 1 set |
| X/Y/Z Axis Helical Rack | SHAC | China | 1 set |
| Servo Motor & Driver | Inovance | China | Whole set |
| Reducer | Techmech | Germany / Japan design | Whole set |
| Machine Bed | Wavlon Engineered Frame | China-fabricated | Whole set |
| Pneumatic System | AirTAC / SMC | China | Whole set |
| Electric Proportional Valve | SMC | Japan | 1 set |
| Cooling System (Chiller) | S&A | China | Whole set |
| Electrical Components | Schneider / DELIXI | China | 1 set |
| Loading System | Semi-Auto (TubeCut Double Chuck) / Full-Auto Pneumatic Roller (TubeCut Triple Chuck) | China | 1 set |

Per LD reference notes: if a customer requests configuration changes away from the specified brands/models above, the resulting impact on whole-machine performance is the customer's responsibility. Standard whole-machine warranty period is 1 year (excluding consumables such as transmission fiber, lenses, and damage from natural disaster, war, or misuse).

# 6. LD Laser Cross-Reference

Maps each Wavlon TubeCut model to its closest LD Laser catalog equivalent for engineering and sourcing traceability.

| Wavlon Model | Chucks | Max ⌀ | Power Range | Closest LD Reference | LD Range / Max ⌀ |
| --- | --- | --- | --- | --- | --- |
| TDC-120 | 2 | 120 mm | 1.5–3 kW | T5 Series (code 6012) | 1.5–12 kW / ⌀360mm |
| TDC-160 | 2 | 160 mm | 2–6 kW | T5 Series (code 6016) | 1.5–12 kW / ⌀360mm |
| TDC-240 | 2 | 240 mm | 4–8 kW | T5 Series (code 6024) | 1.5–12 kW / ⌀360mm |
| TDC-360 | 2 | 360 mm | 6–12 kW | T5 Series (code 6036) | 1.5–12 kW / ⌀360mm |
| TTC-360 | 3 | 360 mm | 6–12 kW | T8 Series (code 12036) | 6–30 kW / ⌀830mm |
| TTC-520 | 3 | 520 mm | 12–20 kW | T8 Series (code 12052) | 6–30 kW / ⌀830mm |
| TTC-680/570 | 3 | 680/570 mm | 20–30 kW | T8 Series (closest: code 12064/12083) | 6–30 kW / ⌀830mm |

# 7. Site & Installation Requirements

Adapted from the LD Laser reference installation guidelines; applies to all TubeCut Double Chuck and TubeCut Triple Chuck models.

- Ground: installation surface flatness within ±5mm; concrete thickness on the full installation footprint not less than 150mm.

- Foundation: standard plan drawing required if concrete thickness is below 150mm; foundation plan drawing applies above 150mm.

- Space: minimum 2,500mm clearance between the machine's auxiliary side / rear and the workshop wall; laser source at least 1,000mm from any wall; fire extinguishers and clear evacuation paths required.

- Environment: dry, non-corrosive, dust-free, well-ventilated; ambient temperature +4°C to +40°C; humidity below 75%; no direct sunlight; no water seepage.

- Vibration: no vibration source within 50m of the equipment; install an anti-seismic trench if unavoidable.

- Power: total supply capacity sized to laser power rating; voltage stability within 5%; three-phase imbalance below 2.5%.

- Environmental quality: no airborne particles/vapors that absorb the 1.064µm wavelength (e.g., paint overspray, degreasing vapor) near the cutting zone.

- Grounding: machine bed grounded separately; grounding resistance below 4Ω; copper grounding rod/plate of at least 1.5m driven into salted earth, bonded with multi-strand copper wire sized to the machine's total supply cross-section.

# 8. Notes, Assumptions & Open Items

- Power tier options (e.g., 1.5/2/3kW for TDC-120) are proposed groupings based on LD's stated 1.5–12kW (T5) and 6–30kW (T8) overall ranges — confirm exact discrete wattages available from the laser source supplier (Raycus/MAX) before publishing external pricing.

- Acceleration figure for TubeCut Triple Chuck (≈1.0G) is an engineering estimate; LD's published T8 catalog page did not display a numeric acceleration figure at the time of extraction — confirm with supplier.

- Machine footprint and gross weight for all models except TDC-120 are scaled estimates from the single grounded LD/Wavlon reference datasheet (S5-6012-3000W) — treat as indicative only until confirmed.

- TTC-680/570 flagship diameter was specified directly by Wavlon management and is a custom target that does not exactly match any single LD T8 catalog code — see Section 4.4.

- This document does not yet reflect a published Wavlon website page; it is an internal reference for sales engineering, quoting, and eventual website/CRM (Lovable) data entry.

— End of Document —